# PPT (Paratrike) Basic Licence — Requirements and Documents

Apex Adventures (Paramotor SA (Pty) Ltd, SACAA DTO 0012) · Paratrike (PPT) only, based on the PPG Basic licence · Compiled 26 Oct 2026 from CAR Part 62 Subpart 8, SA-CATS 62 Appendices R62.35 / R62.37, the SAHPA Training and Procedures Manual (TPM) and SAHPA's 2025 forms.

## 1. Who this is for

Pilots who want to fly a paratrike (wheeled powered paraglider) solo. SAHPA treats a paratrike as a powered paraglider with a tricycle undercarriage, and trike training is a conversion from PPG. The Basic requirements below are therefore the PPG Basic requirements plus the paratrike conversion. No separate PPT Basic figures were found, so confirm with SAHPA before publishing.

## 2. Before you start

- PG Basic completed and PPG training under way or completed (PPG Basic requirements in section 3)

- Registered with SAHPA as a student; copy of ID or passport on file

- Medical fitness declaration (SAHPA Medical Fitness form)

- Restricted radiotelephony operator's certificate

- Theory exams passed, then the skills test within 90 days of the exam and within 60 days before you apply (CAR 62.08.5)

## 3. Experience requirements (PPG Basic, SA-CATS R62.35)

| Requirement | Basic |
| --- | --- |
| PG Basic licence | Completed first |
| High flights | 35 high flights of at least 60 seconds each |
| PPG flights | 10 flights of at least 60 seconds |
| Airtime on PPG | At least 4 hours (3 hours for a conversion course, TPM) |
| Landings | 5 safe power-on landings within 5 m of a target and 5 safe power-off landings within 7 m, into wind |

Confirm with your instructor whether trike flights count towards the PPG flights and hours, or whether flights on a foot-launched PPG are also required.

## 4. Paratrike conversion (SAHPA TPM 5.12)

You must show that you can:

- Assess conditions for a safe trike launch (weather, field, wing weight range and rating)

- Pre-flight the trike and canopy: motor/trike connection, canopy layout, avoiding wheel and line entanglement

- Centre the canopy overhead mainly with the brake toggles and minimal nose-wheel steering

- Use power to keep the canopy pressurised during launch

- Complete 5 successful no-wind launches

- Understand and control the higher approach speed caused by greater wing loading

- Flare correctly timed on landing

- Complete 5 successful landings with proper canopy control and deflation

Tandem flights are only allowed after a tandem rating.

Equipment: paramotor/trike with protective cage and harness; a wing approved under SAHPA rules; air-band radio and headset where aviation rules require one (any flight above 1500 ft AGL, within 10 nm of an active airfield or controlled airspace, or along a known traffic route); an altimeter, vario or GPS to monitor height and climb rate. Boots with ankle support and a reserve parachute are recommended, not compulsory.

## 5. Theory exams

- Pass both the PG and PPG written exams (TPM), pass mark 85% (SA-CATS R62.37)

- Subjects: General, Airflow, Aerodynamics, Flight Control, Air Rules, Airmanship, and others pertaining to powered flight

## 6. Practical / skills test

All tasks in SA-CATS-FCL 62.08.5.1, flown on the paratrike, plus the PPG Basic items: pre-flight of canopy, hang-point and motor; site analysis and engine-out planning; at least two ways to shut the motor off; 3 of the last 4 forward inflations and 3 of the last 4 reverse inflations with launch; two minutes of controlled kiting; co-ordinated 90, 180 and 360 degree turns; and the landing standards in section 3.

## 7. Documents to submit

| # | Document | Notes |
| --- | --- | --- |
| 1 | SAHPA membership application form | Completed and signed by the pilot and a Grade A or B instructor |
| 2 | Medical Fitness form | Per SA-CATS |
| 3 | Marked and passed PG and PPG written exams | 85% or higher |
| 4 | Restricted radiotelephony operator's certificate | Copy |
| 5 | Training Proficiency Card and Practical Requirements Card | Completed and signed |
| 6 | Basic Licence Theory Checklist | Completed and signed |
| 7 | Logbook copy | Signed by a Grade A or B instructor; paratrike conversion flights and landings clearly shown |
| 8 | Logbook endorsement for the paratrike conversion | Signed by your instructor |
| 9 | CA 62.16 application form | From SAHPA Forms & Checklists. Flights and hours must match your logbook |
| 10 | CA 62.15 skills test form | Completed and signed after the skills test, flown on the paratrike |
| 11 | CA 183-540 consent form | Signed |
| 12 | Certified copy of ID or passport, and 2 colour ID photographs | Follow SAHPA's ID & Passport Specifications sheet |
| 13 | Membership and licence fees with proof of payment | Current SAHPA fee schedule |

## 8. Steps

- Complete PG Basic and the PPG Basic requirements.

- Complete the paratrike conversion and get the logbook endorsement.

- Pass the PG and PPG exams (85%).

- Fly the skills test on the paratrike within 90 days of the exam.

- Your instructor signs the cards, checklist and logbook.

- Submit the documents above to SAHPA within 60 days of the skills test.

- SAHPA submits to SACAA; upload your SACAA licence in the Flight Hub when it is issued.

## 9. Source notes

- CAR Part 62 Subpart 8; SA-CATS 62 Appendices R62.35 (item 1(ii), PPG Basic) and R62.37; SAHPA TPM Part 5 (5.10 and 5.12); SAHPA Forms & Checklists page (2025 packs).

- SAHPA's own manuals use "PPT" for both powered parachutes and paratrikes. This sheet covers the wheeled paratrike only. The powered parachute (PPC) Basic licence is different: 3 hours of dual training and 7 hours of solo flight after the PG Basic requirements.

- Part 106 (Hang-gliding and Paragliding) was published as a draft for comment in May 2022; its final status was not confirmed.

- SA-CATS 62 Appendix R62.38 (skills test requirements) was not read in full. No Basic figures specific to paratrikes were found.